FINAL UVM VERIFICATION PROJECT

AXI4 Full Burst Memory and FIFO Peripheral

# 1. Objective

Develop a complete UVM verification solution for the provided AXI4 Full memory-mapped peripheral. The trainee must independently create the verification plan and derive the complete UVM architecture, stimulus, checking, coverage and regression strategy.

# 2. DUT Interface

| Signal / Group | Width | Purpose |
| --- | --- | --- |
| ACLK / ARESETn | 1 | Clock / active-low reset |
| AW* | ID=4, ADDR=16 | Write-address channel |
| W* | DATA=32, STRB=4 | Write-data channel |
| B* | ID=4, RESP=2 | Write-response channel |
| AR* | ID=4, ADDR=16 | Read-address channel |
| R* | ID=4, DATA=32, RESP=2 | Read-data channel |
| IRQ | 1 | Peripheral interrupt |

# 3. DUT Functional Blocks

4 KiB byte-addressable RAM.

Register bank.

16-entry x 32-bit FIFO.

Interrupt controller.

Configurable channel delay/backpressure.

AXI4 transaction IDs.

Queued read/write addresses.

Burst, byte-strobe and error-response logic.

# 4. Address / Register Map

| Address | Block / Register | Description |
| --- | --- | --- |
| 0x0000-0x0FFF | RAM | 4 KiB memory |
| 0x1000 | CTRL | Peripheral control |
| 0x1004 | STATUS | Read-only status |
| 0x1008 | INT_EN | Interrupt enable |
| 0x100C | INT_STATUS | W1C interrupt status |
| 0x1010 | FIFO_DATA | FIFO write/read |
| 0x1014 | FIFO_STATUS | Read-only FIFO status |
| 0x1018 | DELAY_CFG | Read/write channel delay limits |
| 0x101C | TXN_COUNT | Read-only completed transaction count |
| 0x2000-0x2FFF | Error window | SLVERR |
| Other | Unmapped | DECERR |

# 5. Minimum DUT Behavior

32-bit AXI4 Full slave, 16-bit address and 4-bit transaction ID.

Support FIXED and INCR bursts.

Maximum burst length is 16 beats.

Supported transfer sizes are 1, 2 and 4 bytes.

WRAP and unsupported burst/size combinations return an error.

INCR bursts crossing a 4 KiB boundary are illegal.

AW, W, B, AR and R channels are independent.

The slave may insert channel backpressure/response delay.

WSTRB controls byte updates.

WLAST and RLAST must identify the final beat.

Read/write responses return the matching ID.

The DUT can accept multiple queued AW and AR requests.

FIFO overflow/underflow and illegal FIFO usage are error conditions.

Interrupt status is W1C.

# 6. Project Path

Produce the verification plan before the environment is considered complete.

Define the transaction model, agent architecture, sequence strategy, monitor reconstruction, scoreboard/reference model, assertions, functional coverage, test list, regression and exit criteria.

AXI channel independence, burst behavior, IDs, backpressure and error handling must be verified.

The monitor/checking path must be independent of the driver.

The environment must be self-checking.

Directed, constrained-random, negative and stress verification are expected.

Technical ambiguities are to be discussed during mentor reviews.

# 7. Submission

Complete verification repository.

Verification plan.

UVM testbench.

Regression results.

Functional coverage report.

Final verification report.

Bug reports for defects found.